EMBEDDED AND IoT SYSTEMS
PRACTICAL EXPERIMENTS

AIM • SOFTWARE REQUIRED • PROCEDURE • PROGRAM

Note: Update Wi-Fi credentials, API keys, Blynk template details, and pin connections for your own hardware. Install the required libraries through Arduino IDE Library Manager. Use only low-voltage loads for relay demonstrations.

# 1. Temperature Sensor Data Display on LCD

## AIM

To write a program to read temperature data from an LM35 sensor and display the value on a 16×2 LCD.

## SOFTWARE / HARDWARE REQUIRED

Arduino IDE; Arduino Uno; LM35 temperature sensor; 16×2 I2C LCD; jumper wires and breadboard.

## PROCEDURE

Connect LM35 VCC to 5V, GND to GND, and OUT to A0 on the Arduino Uno.

Connect the I2C LCD to 5V, GND, SDA (A4), and SCL (A5).

Install the LiquidCrystal_I2C library and select the correct LCD I2C address (commonly 0x27 or 0x3F).

Enter and upload the program to the Arduino Uno.

Observe the temperature value displayed on the LCD.

## PROGRAM

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x27, 16, 2);

const int sensorPin = A0;

void setup() {

lcd.init();

lcd.backlight();

}

void loop() {

int raw = analogRead(sensorPin);

float voltage = raw * (5.0 / 1023.0);

float temperatureC = voltage * 100.0; // LM35: 10 mV/°C

lcd.setCursor(0, 0);

lcd.print("Temperature:");

lcd.setCursor(0, 1);

lcd.print(temperatureC, 1);

lcd.print((char)223);

lcd.print("C     ");

delay(1000);

}

# 2. Sensor Data Logging to an IoT Cloud Platform

## AIM

To upload sensor data to an IoT cloud platform and visualize it on a dashboard.

## SOFTWARE / HARDWARE REQUIRED

Arduino IDE; ESP32; DHT11 sensor; ThingSpeak account; Wi-Fi connection; DHT sensor library and Adafruit Unified Sensor library.

## PROCEDURE

Connect the DHT11 data pin to GPIO 4, VCC to 3.3V, and GND to GND (follow the module's pin labels).

Create a ThingSpeak channel with fields for temperature and humidity.

Copy the channel Write API Key and enter it, along with Wi-Fi credentials, in the program.

Install the DHT sensor library and upload the program to the ESP32.

Open the ThingSpeak channel and verify that readings appear in the charts.

## PROGRAM

#include <WiFi.h>

#include <HTTPClient.h>

#include "DHT.h"

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASSWORD";

String writeKey = "YOUR_THINGSPEAK_WRITE_API_KEY";

void setup() {

Serial.begin(115200);

dht.begin();

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) delay(500);

}

void loop() {

float t = dht.readTemperature();

float h = dht.readHumidity();

if (WiFi.status() == WL_CONNECTED && !isnan(t) && !isnan(h)) {

HTTPClient http;

String url = "http://api.thingspeak.com/update?api_key=" + writeKey +

"&field1=" + String(t) + "&field2=" + String(h);

http.begin(url);

http.GET();

http.end();

Serial.printf("Temp: %.1f C, Humidity: %.1f %%\\n", t, h);

}

delay(20000); // ThingSpeak free channels commonly require 15 s between updates

}

# 3. Sending Sensor Data to a Mobile App Using Blynk

## AIM

To transmit sensor readings from an IoT device to a mobile application using the Blynk platform.

## SOFTWARE / HARDWARE REQUIRED

Arduino IDE; ESP32; DHT11 sensor; Blynk IoT mobile app/dashboard; Wi-Fi; Blynk library and DHT library.

## PROCEDURE

Connect the DHT11 data pin to GPIO 4, with power and ground connected correctly.

Create a Blynk template/device and configure datastreams V0 (temperature) and V1 (humidity).

Copy the Blynk template ID, device name, and authentication token into the program.

Install the Blynk and DHT libraries, then upload the program to the ESP32.

Run the Blynk app/dashboard and verify the temperature and humidity values.

## PROGRAM

#define BLYNK_TEMPLATE_ID "YOUR_TEMPLATE_ID"

#define BLYNK_TEMPLATE_NAME "Sensor Monitor"

#define BLYNK_AUTH_TOKEN "YOUR_DEVICE_AUTH_TOKEN"

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

#include "DHT.h"

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

char ssid[] = "YOUR_WIFI_NAME";

char pass[] = "YOUR_WIFI_PASSWORD";

BlynkTimer timer;

void sendSensor() {

float t = dht.readTemperature();

float h = dht.readHumidity();

if (!isnan(t) && !isnan(h)) {

Blynk.virtualWrite(V0, t);

Blynk.virtualWrite(V1, h);

}

}

void setup() {

Serial.begin(115200);

dht.begin();

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

timer.setInterval(2000L, sendSensor);

}

void loop() {

Blynk.run();

timer.run();

}

# 4. Secure Communication for an IoT Device Using Wi-Fi

## AIM

To demonstrate secure communication between an IoT device and a server using Wi-Fi and HTTPS with certificate validation.

## SOFTWARE / HARDWARE REQUIRED

Arduino IDE; ESP32; Wi-Fi network; HTTPS test endpoint/server; WiFiClientSecure and HTTPClient libraries (included with ESP32 board package).

## PROCEDURE

Connect the ESP32 to the computer and install the ESP32 board package in Arduino IDE.

Configure the Wi-Fi credentials and the trusted root CA certificate for the HTTPS server in the program.

Upload the program to the ESP32.

The device connects to Wi-Fi and sends a sample request to the HTTPS endpoint.

Check the serial monitor for the HTTP response. Certificate validation should remain enabled.

## PROGRAM

#include <WiFi.h>

#include <WiFiClientSecure.h>

#include <HTTPClient.h>

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASSWORD";

const char* serverUrl = "https://example.com/";

const char* rootCACertificate = R"EOF(

-----BEGIN CERTIFICATE-----

PASTE_THE_SERVER_TRUSTED_ROOT_CA_CERTIFICATE_HERE

-----END CERTIFICATE-----

)EOF";

void setup() {

Serial.begin(115200);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) delay(500);

WiFiClientSecure client;

client.setCACert(rootCACertificate); // Validates server certificate

HTTPClient https;

if (https.begin(client, serverUrl)) {

int code = https.GET();

Serial.printf("HTTPS response code: %d\\n", code);

if (code > 0) Serial.println(https.getString());

https.end();

}

}

void loop() {

}

# 5. Simple IoT-Based Automation System

## AIM

To design and implement an IoT-based automation system for remotely switching a low-voltage device through a mobile dashboard.

## SOFTWARE / HARDWARE REQUIRED

Arduino IDE; ESP32; relay module; low-voltage LED/load; Blynk IoT app; Wi-Fi; Blynk library.

## PROCEDURE

Connect the relay input to GPIO 5, and connect the relay module's VCC and GND as specified by its manufacturer.

Connect a low-voltage load to the relay contacts; do not handle exposed mains wiring.

Create a Blynk device and add a switch datastream on virtual pin V0.

Enter the Blynk credentials and Wi-Fi details in the program.

Upload the program and operate the switch in Blynk to turn the load on and off.

## PROGRAM

#define BLYNK_TEMPLATE_ID "YOUR_TEMPLATE_ID"

#define BLYNK_TEMPLATE_NAME "IoT Automation"

#define BLYNK_AUTH_TOKEN "YOUR_DEVICE_AUTH_TOKEN"

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

char ssid[] = "YOUR_WIFI_NAME";

char pass[] = "YOUR_WIFI_PASSWORD";

const int relayPin = 5;

BLYNK_WRITE(V0) {

int state = param.asInt();

digitalWrite(relayPin, state ? LOW : HIGH); // Many relay modules are active-LOW

}

void setup() {

pinMode(relayPin, OUTPUT);

digitalWrite(relayPin, HIGH);

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

}

void loop() {

Blynk.run();

}

# 6. LED Blinking Using Arduino IDE

## AIM

To write and execute an Arduino program to blink an LED at regular intervals.

## SOFTWARE / HARDWARE REQUIRED

Arduino IDE; Arduino Uno; LED; 220-ohm resistor; breadboard; jumper wires; USB cable.

## PROCEDURE

Connect the LED anode through a 220-ohm resistor to digital pin 13; connect the cathode to GND.

Open Arduino IDE and select Arduino Uno and the correct port.

Enter the program and click Verify to compile it.

Upload the program to the board.

Observe the LED switching on and off every second.

## PROGRAM

const int ledPin = 13;

void setup() {

pinMode(ledPin, OUTPUT);

}

void loop() {

digitalWrite(ledPin, HIGH);

delay(1000);

digitalWrite(ledPin, LOW);

delay(1000);

}

# 7. Cloud IoT Platform Setup for Real-Time Data Logging

## AIM

To configure a cloud IoT platform and demonstrate real-time logging and visualization of sensor data.

## SOFTWARE / HARDWARE REQUIRED

Arduino IDE; ESP32; DHT11 sensor; ThingSpeak account; Wi-Fi; DHT sensor library and Adafruit Unified Sensor library.

## PROCEDURE

Create a ThingSpeak channel with temperature and humidity fields.

Connect the DHT11 sensor to GPIO 4 of the ESP32.

Copy the channel Write API Key and configure Wi-Fi credentials in the program.

Upload the program to the ESP32 and allow it to send readings at regular intervals.

View the channel charts to confirm that cloud data logging is working.

## PROGRAM

#include <WiFi.h>

#include <HTTPClient.h>

#include "DHT.h"

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASSWORD";

const char* apiKey = "YOUR_THINGSPEAK_WRITE_API_KEY";

void setup() {

Serial.begin(115200);

dht.begin();

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) delay(500);

}

void loop() {

float t = dht.readTemperature();

float h = dht.readHumidity();

if (WiFi.status() == WL_CONNECTED && !isnan(t) && !isnan(h)) {

WiFiClient client;

HTTPClient http;

String url = String("http://api.thingspeak.com/update?api_key=") + apiKey +

"&field1=" + String(t) + "&field2=" + String(h);

http.begin(client, url);

Serial.print("Cloud update result: ");

Serial.println(http.GET());

http.end();

}

delay(20000);

}

# 8. IoT System Integrating Sensors, Controller, and Cloud Connectivity

## AIM

To implement an IoT system that reads environmental data using sensors, processes it with a controller, and uploads it to a cloud platform.

## SOFTWARE / HARDWARE REQUIRED

Arduino IDE; ESP32; DHT11 sensor; ThingSpeak; Wi-Fi; DHT sensor library and Adafruit Unified Sensor library.

## PROCEDURE

Connect the DHT11 sensor data pin to GPIO 4 on the ESP32.

Create a ThingSpeak channel with fields for temperature and humidity.

Enter the Wi-Fi credentials and ThingSpeak Write API Key in the program.

Upload the program to the ESP32.

Check the serial monitor and cloud dashboard to confirm successful sensor reading and upload.

## PROGRAM

#include <WiFi.h>

#include <HTTPClient.h>

#include "DHT.h"

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASSWORD";

const char* apiKey = "YOUR_THINGSPEAK_WRITE_API_KEY";

void setup() {

Serial.begin(115200);

dht.begin();

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) delay(500);

Serial.println("Wi-Fi connected");

}

void loop() {

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

if (!isnan(temperature) && !isnan(humidity) &&

WiFi.status() == WL_CONNECTED) {

HTTPClient http;

String url = String("http://api.thingspeak.com/update?api_key=") + apiKey +

"&field1=" + String(temperature) + "&field2=" + String(humidity);

http.begin(url);

int result = http.GET();

Serial.printf("T=%.1f C, H=%.1f %%, HTTP=%d\\n",

temperature, humidity, result);

http.end();

}

delay(20000);

}

# 9. IoT-Based Smart Parking Management System

## AIM

To design and implement an IoT-based smart parking system that detects slot occupancy and sends parking status to a cloud dashboard.

## SOFTWARE / HARDWARE REQUIRED

Arduino IDE; ESP32; two HC-SR04 ultrasonic sensors; LEDs and 220-ohm resistors; ThingSpeak account; Wi-Fi; jumper wires.

## PROCEDURE

Connect the two ultrasonic sensors' TRIG pins to GPIO 5 and 18, and ECHO pins to GPIO 19 and 21 respectively. Use a voltage divider on each ECHO line to reduce 5V to a safe 3.3V for ESP32.

Connect an LED with a 220-ohm resistor to GPIO 22 and another to GPIO 23.

Create a ThingSpeak channel with fields for slot 1 and slot 2 status.

Enter Wi-Fi credentials and the channel Write API Key in the program.

Upload the program and place an object/vehicle in front of each sensor to simulate an occupied slot.

Observe the LEDs and cloud dashboard to confirm that the slot status changes.

## PROGRAM

#include <WiFi.h>

#include <HTTPClient.h>

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASSWORD";

const char* apiKey = "YOUR_THINGSPEAK_WRITE_API_KEY";

const int trig1 = 5, echo1 = 19;

const int trig2 = 18, echo2 = 21;

const int led1 = 22, led2 = 23;

float distanceCM(int trigPin, int echoPin) {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH, 30000);

if (duration == 0) return 999;

return duration * 0.0343 / 2.0;

}

void setup() {

Serial.begin(115200);

pinMode(trig1, OUTPUT); pinMode(echo1, INPUT);

pinMode(trig2, OUTPUT); pinMode(echo2, INPUT);

pinMode(led1, OUTPUT); pinMode(led2, OUTPUT);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) delay(500);

}

void loop() {

// A reading below 15 cm means occupied; adjust for your setup.

int slot1 = distanceCM(trig1, echo1) < 15 ? 1 : 0;

int slot2 = distanceCM(trig2, echo2) < 15 ? 1 : 0;

digitalWrite(led1, slot1 ? HIGH : LOW);

digitalWrite(led2, slot2 ? HIGH : LOW);

Serial.printf("Slot 1: %s, Slot 2: %s\\n",

slot1 ? "Occupied" : "Free",

slot2 ? "Occupied" : "Free");

if (WiFi.status() == WL_CONNECTED) {

HTTPClient http;

String url = String("http://api.thingspeak.com/update?api_key=") + apiKey +

"&field1=" + String(slot1) + "&field2=" + String(slot2);

http.begin(url);

http.GET();

http.end();

}

delay(20000);

}

# General Setup Notes

ESP32 pin assignments and voltage levels vary by board. Check the board pinout before wiring.

For DHT11 experiments, install the DHT sensor library and Adafruit Unified Sensor library from Arduino IDE Library Manager.

For Blynk experiments, install the Blynk library and create the template, device, datastreams, and dashboard widgets before uploading.

For ThingSpeak experiments, replace the placeholder Wi-Fi credentials and Write API Key. Keep the update interval at least 15 seconds; this document uses 20 seconds.

For the LCD experiment, change address 0x27 to 0x3F if required by your I2C LCD module.

For the secure HTTPS experiment, replace the placeholder CA text with the actual trusted root CA certificate for the endpoint. Do not use setInsecure(), because it disables certificate validation.

For the parking experiment, ESP32 GPIO pins are 3.3V tolerant only. Reduce each HC-SR04 ECHO signal from 5V to 3.3V using a voltage divider or suitable level shifter.

Never connect an Arduino/ESP32 directly to mains voltage. Use a properly enclosed, isolated relay module and qualified supervision for mains-powered equipment.